# objective:

- Execute instructions to create delays using subroutines.

- Perform communication with shift registers.

# references:

- Lab manual chapter 1-2.

# EXPERIMENT 1:

- Use the following program:

.include "m324PAdef.inc" .org 00 ldi r16,0x01 out DDRA, r16 start: sbi PORTA,PINA0 cbi PORTA, PINA0 rjmp start

Connect PA0 to a measurement channel on the TEST STATION and measure pulse forms using an oscilloscope.

# EXPERIMENT 2:

- Write a subroutine Delay1ms and use it to write a program to generate a 1KHz square wave on PA0.

- Use this subroutine to write subroutines Delay10ms, Delay100ms, and Delay1s.

- Use the Delay1s subroutine to write a program to blink/turn off an LED connected to PA0.

# EXPERIMENT 3:

- Connect the necessary signals from an AVR port to the control signals of the shift register on header J13. Connect the output of the shift register to a LED bar.

- Using the sample programs from the experiment guide, write a program to create a gradually lit LED effect from left to right, then gradually turn them off from left to right after every 500ms.

# EXPERIMENT 1:

- Answer the following questions:

  - Capture a pulse waveform on PA0.

  - What is the frequency, duration of the high signal, and duration of the low signal?

By using a simulated oscilloscope, we can determine the clock period (T) is 6 microseconds (6 μs), we can calculate the frequency and durations more accurately.

f= 1/T = 166.67 kHz

Duration of the high signal: 26μs -24μs=2μs

Duration of the low signal: 30μs -26μs=4μs

  - Explain the measured results.

The high and low signals are set by the sbi instruction, and it is cleared by the cbi instruction. These results can be attributed to the execution time of the instructions in the microcontroller and the time it takes to change the state of the output pin. The specific timings can vary depending on the microcontroller's architecture and the compiler used to generate the code.

# EXPERIMENT 2:

- Answer the following questions:

  - How to calculate the number of machine cycles needed to execute the Delay1ms subroutine. Present a simulation image.

.cseg .org 0x00 START: SBI DDRA, 0 MAIN: CBI PORTA,0 CALL DELAY_1MS SBI PORTA,0 CALL DELAY_1MS JMP MAIN DELAY_1MS: LDI R21,8 ;1MC L1: LDI R20,250 ;1MC L2: DEC R20 ;1MC NOP ;1MC BRNE L2 ; 2/1MC DEC R21 ;1MC BRNE L1 ;2/1MC RET ;4MC

The above is an example of creating a 50Hz frequency signal on pin PA0. The subprogram

DELAY_1MS generates a 1ms delay with an 8MHz clock, where 1MC (Machine Cycle)

equals 0.125μs, corresponding to 8000MC x 0.125 = 1ms.

  - Image of a 1KHz square wave on PA0.

- Source code for 2.c with comments.

.ORG 0x00 LOOP: SBI PORTA, 0 RCALL DL_1SEC ;Call the delay function for 1s CBI PORTA, 0 RCALL DL_1SEC RJMP LOOP DL_1SEC: LDI R18, 10 C: RCALL DL_100MSEC ;Call the delay function for 100ms DEC R18 BRNE C RET DL_100MSEC: LDI R17, 10 B: RCALL DL_10MSEC ;;Call the delay function for 10ms DEC R17 BRNE B RET DL_10MSEC: LDI R16, 10 A: RCALL DL_1MSEC DEC R16 BRNE A RET DL_1MSEC: LDI R21, 80 LP2: LDI R20, 250 LP1: NOP DEC R20 BRNE LP1 DEC R21 BRNE LP2 RET

# EXPERIMENT 3:

- Answer the following questions:

  - Describe the connections on the experimental kit.

Choosing JTAG, using 8x8 BUS to connect port J36 from Shift Register directly to BARLED Connect PB0 to DSI Connect PB1 to Latch Connect PB2 to CLK Connect PB3 to nCLR

  - According to the datasheet of 74HC595, what is the highest clock frequency it can operate at?

  - How do you expand the display to 16 LEDs?

- Source code with comments.

.include "m324padef.inc" ; Include Atmega324pa definitions .def shiftData = r20 ; Define the shift data register .equ clearSignalPort = PORTB ; Set clear signal port to PORTB .equ clearSignalPin = 3 ; Set clear signal pin to pin 0 of PORTB .equ shiftClockPort = PORTB ; Set shift clock port to PORTB .equ shiftClockPin = 2 ; Set shift clock pin to pin 1 of PORTB .equ latchPort = PORTB ; Set latch port to PORTB .equ latchPin = 1 ; Set latch pin to pin 0 of PORTB .equ shiftDataPort = PORTB ; Set shift data port to PORTB .equ shiftDataPin = 0 ; Set shift data pin to pin 3 of PORTB main: call initport ldi shiftData,0x55 call cleardata call shiftoutdata stop: jmp stop ; Initialize ports as outputs initport: ldi r24,(1<<clearSignalPin)¦(1<<shiftClockPin)¦(1<<latchPin)¦(1<<shiftDataPin) out DDRB, r24 ; Set DDRB to output ret ldi shiftData,0x55 cleardata: cbi clearSignalPort, clearSignalPin ; Set clear signal pin to low ; Wait for a short time sbi clearSignalPort, clearSignalPin ; Set clear signal pin to high ret ; Shift out data shiftoutdata: cbi shiftClockPort, shiftClockPin ; ldi r18, 8 ; Shift 8 bits shiftloop: sbrc shiftData, 7 ; Check if the MSB of shiftData is 1 sbi shiftDataPort, shiftDataPin ; Set shift data pin to high sbi shiftClockPort, shiftClockPin ; Set shift clock pin to high lsl shiftData ; Shift left cbi shiftClockPort, shiftClockPin ; Set shift clock pin to low cbi shiftDataPort, shiftDataPin ; Set shift data pin to low dec r18 brne shiftloop ; Latch data sbi latchPort, latchPin ; Set latch pin to high CALL DELAY_1S cbi latchPort, latchPin ; Set latch pin to low DEC R16 BRNE shiftoutdata RET DELAY_500mS: LDI R16,250 L2: LDI R17,250 L1: DEC R17 NOP NOP NOP NOP NOP BRNE L1 DEC R16 BRNE L2 RET