# objective:

- Understand how to debounce a button.

- Understand how to communicate with an LCD.

- Understand how to communicate with a single button.

- Understand how to communicate with a matrix keypad.

# Reference:

- Lab manual chapter 1, 2 , 3 ,6

# EXPERIMENT 1:

- Connect one AVR PORT to J33 (LCD control header) on the experimental kit.

- Use sample programs from the experiment guide, write a program to initialize the LCD and display the following on the LCD (XX is the group number):

EX VXL-AVR GROUP: XX

# EXPERIMENT 2:

- Connect a switch to a port pin of AVR, connect a BAR LED module to another AVR port, and connect an LCD to a port of AVR.

- Write a program to count the number of button presses and display  the result on the BAR LED and LCD (without debouncing).

- Add button debounce functionality to the program.

- Execute the program, press/release the button, and observe the results.

# EXPERIMENT 3:

- Connect signals from one port of AVR to the matrix keypad module, and connect the BAR LED and LCD to two different ports of AVR.

- Write a subroutine SCANKEY to scan the matrix keypad and return a value from 0x0 to 0xF corresponding to the pressed key's code. If no key is pressed, return 0xFF. The returned value should be stored in R24.

- Using this subroutine, write a program to scan the keypad and display the read value on the BAR LED and LCD.

- Execute the program and observe the results

# EXPERIMENT 1:

- Answer the following questions:

  - How does the LCD distinguish between command and data?

To send a command to the LCD, you place the command code on the DATA signals, set RS to 0 to indicate the command register, set RW to 0 for write mode, and initially keep EN at 0. Then, you generate a pulse on EN by toggling it from 0 to 1 and back to 0 to transmit the command. To write data to the LCD for display, you set the data on the DATA signals, set RS to 1 to indicate the data register, set RW to 0 for write mode, and initially keep EN at 0. Again, you create a pulse on EN by toggling it from 0 to 1 and back to 0 to send the data to the data register. In summary, the LCD distinguishes between commands and data based on the RS signal. When RS is 0, it expects a command, and when RS is 1, it receives data for display.

  - Besides checking the BUSY bit, what other method ensures that the LCD is ready to receive data/command?

By reading the command register, you may determine if the LCD is prepared to accept a command. Set the RS signal to 0 to choose the command register, the RW signal to 1 to choose the read mode, and the EN signal to 0 to start. Next, you establish a pulse. EN is activated by switching the EN signal from 0 to 1 and back again. The LCD may now be seen. The data pins will have data available.

  - Describe the connections on the experimental kit.

JTAG is chosen, and an 8x8 BUS is used to link the PORTA directly to the LCD. Connect PA0 to LCD_RS Connect PA1 to LCD_RW Connect PA2 to LCD_EN Connect PA4…PA7 to LCD_DB4…LCD_DB7

  - Source code for the program with comments.

.include "m324padef.inc" ; Include Atmega324pa definitions .org 0x0000 ; interrupt vector table rjmp reset_handler ; reset ;******************************* Program ID ********************************* .org INT_VECTORS_SIZE course_name: .db "TN Vi Xu Ly-AVR",0 course_time: .db "HK2 2022-2023",0 reset_handler: call LCD_Init ; display the first line of information ldi ZH, high(course_name) ; point to the information that is to be ldi ZL, low(course_name) call LCD_Send_String ldi r16,1 ldi r17,0 call LCD_Move_Cursor ldi ZH, high(course_time) ; point to the information that is to be ldi ZL, low(course_time) call LCD_Send_String start: rjmp start LCD_Init: ; Set up data direction register for Port A ldi r16, 0b11110111 ; set PA7-PA4 as outputs, PA2-PA0 as output out LCDPORTDIR, r16 ; Wait for LCD to power up call DELAY_10MS call DELAY_10MS ; Send initialization sequence ldi r16, 0x02 ; Function Set: 4-bit interface call LCD_Send_Command ldi r16, 0x28 ; Function Set: enable 5x7 mode for chars call LCD_Send_Command ldi r16, 0x0E ; Display Control: Display OFF, Cursor ON call LCD_Send_Command ldi r16, 0x01 ; Clear Display call LCD_Send_Command ldi r16, 0x80 ; Clear Display call LCD_Send_Command ret .def LCDData = r16 LCD_Send_String: push ZH ; preserve pointer registers push ZL push LCDData ; fix up the pointers for use with the 'lpm' instruction lsl ZL ; shift the pointer one bit left for the rol ZH ; write the string of characters LCD_Send_String_01: lpm LCDData, Z+ ; get a character cpi LCDData, 0 ; check for end of string breq LCD_Send_String_02 ; done ; arrive here if this is a valid character call LCD_Send_Data ; display the character rjmp LCD_Send_String_01 ; not done, send another character ; arrive here when all characters in the message have been sent to the LCD module LCD_Send_String_02: pop LCDData pop ZL ; restore pointer registers pop ZH ret LCD_Move_Cursor: cpi r16,0 ;check if first row brne LCD_Move_Cursor_Second andi r17, 0x0F ori r17,0x80 mov r16,r17 ; Send command to LCD call LCD_Send_Command ret LCD_Move_Cursor_Second: cpi r16,1 ;check if second row brne LCD_Move_Cursor_Exit ;else exit andi r17, 0x0F ori r17,0xC0 mov r16,r17 ; Send command to LCD call LCD_Send_Command LCD_Move_Cursor_Exit: ; Return from function ret .equ LCDPORT = PORTA ; Set signal port reg to PORTA .equ LCDPORTDIR = DDRA ; Set signal port dir reg to PORTA .equ LCDPORTPIN = PINA ; Set clear signal port pin reg to PORTA .equ LCD_RS = PINA0 .equ LCD_RW = PINA1 .equ LCD_EN = PINA2 .equ LCD_D7 = PINA7 .equ LCD_D6 = PINA6 .equ LCD_D5 = PINA5 .equ LCD_D4 = PINA4 DELAY_10MS: LDI R21,80 ;1MC L1: LDI R20,250 ;1MC L2: DEC R20 ;1MC NOP ;1MC BRNE L2 ;2/1MC DEC R21 ;1MC BRNE L1 ;2/1MC RET ;4MC LCD_Send_Command: push r17 call LCD_wait_busy ; check if LCD is busy mov r17,r16 ;save the command ; Set RS low to select command register ; Set RW low to write to LCD andi r17,0xF0 ; Send command to LCD out LCDPORT, r17 nop nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN swap r16 andi r16,0xF0 ; Send command to LCD out LCDPORT, r16 ; Pulse enable pin sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN pop r17 ret LCD_Send_Data: push r17 call LCD_wait_busy ;check if LCD is busy mov r17,r16 ;save the command ; Set RS high to select data register ; Set RW low to write to LCD andi r17,0xF0 ori r17,0x01 ; Send data to LCD out LCDPORT, r17 nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop cbi LCDPORT, LCD_EN ; Delay for command execution ;send the lower nibble nop swap r16 andi r16,0xF0 ; Set RS high to select data register ; Set RW low to write to LCD andi r16,0xF0 ori r16,0x01 ; Send command to LCD out LCDPORT, r16 nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop cbi LCDPORT, LCD_EN pop r17 ret LCD_wait_busy: push r16 ldi r16, 0b00000111 ; set PA7-PA4 as input, PA2-PA0 as output out LCDPORTDIR, r16 ldi r16,0b11110010 ; set RS=0, RW=1 for read the busy flag out LCDPORT, r16 nop LCD_wait_busy_loop: sbi LCDPORT, LCD_EN nop nop in r16, LCDPORTPIN cbi LCDPORT, LCD_EN nop sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN nop andi r16,0x80 cpi r16,0x80 breq LCD_wait_busy_loop ldi r16, 0b11110111 ; set PA7-PA4 as output, PA2-PA0 as output out LCDPORTDIR, r16 ldi r16,0b00000000 ; set RS=0, RW=1 for read the busy flag out LCDPORT, r16 pop r16 ret

# EXPERIMENT 2:

- Answer the following questions:

  - What happens when button debouncing is not implemented?

- Inaccurate inputs and outputs that are unreliable - Flickering display - False triggering When dealing with mechanical key presses, debouncing is important since they produce numerous pulses when pushed or released, which causes erroneous counting. Because mechanical key pushes generate numerous pulses each time a key is pushed or released, not applying debounce can lead to erroneous counting.

  - Describe the connections on the experimental kit.

Choosing JTAG, using 8x8 BUS to connect PORTA directly to LCD Connect PA0 to LCD_RS Connect PA1 to LCD_RW Connect PA2 to LCD_EN Connect PA4…PA7 to LCD_DB4…LCD_DB7 Using 8x8 BUS to connect PORTD to LED Connect PB0 to switch

  - Source code for the program without button debouncing and comments.

.org 0 rjmp start .equ LCDPORT = PORTA ; Set signal port reg to PORTA .equ LCDPORTDIR = DDRA ; Set signal port dir reg to PORTA .equ LCDPORTPIN = PINA ; Set clear signal port pin reg to PORTA .equ LCD_RS = PINA0 .equ LCD_RW = PINA1 .equ LCD_EN = PINA2 .equ LCD_D7 = PINA7 .equ LCD_D6 = PINA6 .equ LCD_D5 = PINA5 .equ LCD_D4 = PINA4 .def LCDData = r16 start: cbi DDRB, 0 ; Set PORTB as input sbi PORTB, 0 ; Enable pull-up resistor ldi r16, 0xff out DDRD, r16 ; Set PORTD as output to LED clr r23 call LCD_Init Sample: call No_Debouncing inc r23 out PORTD, r23 ldi r16, 0x0E ; Display Control: Display OFF, Cursor ON call LCD_Send_Command ldi r16, 0x01 ; Clear Display call LCD_Send_Command ldi r16, 0x80 ; Clear Display call LCD_Send_Command mov r18, r23 ldi r19, 48 ldi r20, 100 clr r21 call Loop_quotient add r21, r19 mov r16, r21 call LCD_Send_Data ldi r20, 10 clr r21 call Loop_quotient add r21, r19 mov r16, r21 call LCD_Send_Data add r18, r19 mov r16, r18 call LCD_Send_Data rjmp Sample LCD_Init: ; Set up data direction register for PortA ldi r16, 0b11110111 ; set PA7-PA4 as outputs, PA2-PA0 as output out LCDPORTDIR, r16 ; Wait for LCD to power up call Delay10ms call Delay10ms ; Send initialization sequence ldi r16, 0x02 ; Function Set: 4-bit interface call LCD_Send_Command ldi r16, 0x28 ; Function Set: enable 5x7 mode for chars call LCD_Send_Command ldi r16, 0x0E ; Display Control: Display OFF, Cursor ON call LCD_Send_Command ldi r16, 0x01 ; Clear Display call LCD_Send_Command ldi r16, 0x80 ; Clear Display call LCD_Send_Command ret LCD_Send_Command: push r17 call LCD_wait_busy ; check if LCD is busy mov r17,r16 ;save the command ; Set RS low to select command register ; Set RW low to write to LCD andi r17,0xF0 ; Send command to LCD out LCDPORT, r17 nop nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN swap r16 andi r16,0xF0 ; Send command to LCD out LCDPORT, r16 ; Pulse enable pin sbi LCDPORT, LCD_EN ;nop ;nop cbi LCDPORT, LCD_EN pop r17 ret LCD_wait_busy: push r16 ldi r16, 0b00000111 ; set PA7-PA4 as input, PA2-PA0 as output out LCDPORTDIR, r16 ldi r16,0b11110010 ; set RS=0, RW=1 for read the busy flag out LCDPORT, r16 nop LCD_wait_busy_loop: sbi LCDPORT, LCD_EN nop nop in r16, LCDPORTPIN cbi LCDPORT, LCD_EN nop sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN nop andi r16,0x80 cpi r16,0x80 breq LCD_wait_busy_loop ldi r16, 0b11110111 ; set PA7-PA4 as output, PA2-PA0 as output out LCDPORTDIR, r16 di r16,0b00000000 ; set RS=0, RW=1 for read the busy flag out LCDPORT, r16 pop r16 ret LCD_Send_Data: push r17 call LCD_wait_busy ;check if LCD is busy mov r17,r16 ;save the command ; Set RS high to select data register ; Set RW low to write to LCD andi r17,0xF0 ori r17,0x01 ; Send data to LCD out LCDPORT, r17 nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop cbi LCDPORT, LCD_EN ; Delay for command execution ;send the lower nibble nop swap r16 andi r16,0xF0 ; Set RS high to select data register ; Set RW low to write to LCD andi r16,0xF0 ori r16,0x01 ; Send command to LCD out LCDPORT, r16 nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop cbi LCDPORT, LCD_EN pop r17 ret Delay10ms: ldi r21,80 ;1MC L1: ldi r20,250 ;1MC L2: dec r20 ;1MC nop ;1MC brne L2 ;2/1MC dec r21 ;1MC brne L1 ;2/1MC ret ;4MC ; Subroutine to obtain the quotient (R21) between dividend (R18) and divisor (R20) Loop_quotient: sub r18, r20 brcc Inc_quotient rjmp Obtain_quotient Inc_quotient: inc r21 rjmp Loop_quotient Obtain_quotient: add r18, r20 ret ; Subroutine to count up without debouncing No_Debouncing: Sampling_ND: sbic PINB, 0 rjmp Sampling_ND Pressed_ND: sbis PINB, 0 rjmp Pressed_ND ret

  - Source code for the program with button debouncing and comments.

.org 0 rjmp start .equ LCDPORT = PORTA ; Set signal port reg to PORTA .equ LCDPORTDIR = DDRA ; Set signal port dir reg to PORTA .equ LCDPORTPIN = PINA ; Set clear signal port pin reg to PORTA .equ LCD_RS = PINA0 .equ LCD_RW = PINA1 .equ LCD_EN = PINA2 .equ LCD_D7 = PINA7 .equ LCD_D6 = PINA6 .equ LCD_D5 = PINA5 .equ LCD_D4 = PINA4 .def LCDData = r16 start: cbi DDRB, 0 ; Set PORTB as input sbi PORTB, 0 ; Enable pull-up resistor ldi r16, 0xff out DDRD, r16 ; Set PORTD as output to LED clr r23 call LCD_Init Sample: call Debouncing inc r23 out PORTD, r23 ldi r16, 0x0E ; Display Control: Display OFF, Cursor ON call LCD_Send_Command ldi r16, 0x01 ; Clear Display call LCD_Send_Command ldi r16, 0x80 ; Clear Display call LCD_Send_Command mov r18, r23 ldi r19, 48 ldi r20, 100 clr r21 call Loop_quotient add r21, r19 mov r16, r21 call LCD_Send_Data ldi r20, 10 clr r21 call Loop_quotient add r21, r19 mov r16, r21 call LCD_Send_Data add r18, r19 mov r16, r18 call LCD_Send_Data rjmp Sample LCD_Init: ; Set up data direction register for PortA ldi r16, 0b11110111 ; set PA7-PA4 as outputs, PA2-PA0 as output out LCDPORTDIR, r16 ; Wait for LCD to power up call Delay10ms call Delay10ms ; Send initialization sequence ldi r16, 0x02 ; Function Set: 4-bit interface call LCD_Send_Command ldi r16, 0x28 ; Function Set: enable 5x7 mode for chars call LCD_Send_Command ldi r16, 0x0E ; Display Control: Display OFF, Cursor ON call LCD_Send_Command ldi r16, 0x01 ; Clear Display call LCD_Send_Command ldi r16, 0x80 ; Clear Display call LCD_Send_Command ret LCD_Send_Command: push r17 call LCD_wait_busy ; check if LCD is busy mov r17,r16 ;save the command ; Set RS low to select command register ; Set RW low to write to LCD andi r17,0xF0 ; Send command to LCD out LCDPORT, r17 nop nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN swap r16 andi r16,0xF0 ; Send command to LCD out LCDPORT, r16 ; Pulse enable pin sbi LCDPORT, LCD_EN ;nop ;nop cbi LCDPORT, LCD_EN pop r17 ret LCD_wait_busy: push r16 ldi r16, 0b00000111 ; set PA7-PA4 as input, PA2-PA0 as output out LCDPORTDIR, r16 ldi r16,0b11110010 ; set RS=0, RW=1 for read the busy flag out LCDPORT, r16 nop LCD_wait_busy_loop: sbi LCDPORT, LCD_EN nop nop in r16, LCDPORTPIN cbi LCDPORT, LCD_EN nop sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN nop andi r16,0x80 cpi r16,0x80 breq LCD_wait_busy_loop ldi r16, 0b11110111 ; set PA7-PA4 as output, PA2-PA0 as output out LCDPORTDIR, r16 ldi r16,0b00000000 ; set RS=0, RW=1 for read the busy flag out LCDPORT, r16 pop r16 ret LCD_Send_Data: push r17 call LCD_wait_busy ;check if LCD is busy mov r17,r16 ;save the command ; Set RS high to select data register ; Set RW low to write to LCD andi r17,0xF0 ori r17,0x01 ; Send data to LCD out LCDPORT, r17 nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop cbi LCDPORT, LCD_EN ; Delay for command execution ;send the lower nibble nop swap r16 andi r16,0xF0 ; Set RS high to select data register ; Set RW low to write to LCD andi r16,0xF0 ori r16,0x01 ; Send command to LCD out LCDPORT, r16 nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop cbi LCDPORT, LCD_EN pop r17 ret Delay10ms: ldi r21,80 ;1MC L1: ldi r20,250 ;1MC L2: dec r20 ;1MC nop ;1MC brne L2 ;2/1MC dec r21 ;1MC brne L1 ;2/1MC ret ;4MC ; Subroutine to obtain the quotient (R21) between dividend (R18) and divisor (R20) Loop_quotient: sub r18, r20 brcc Inc_quotient rjmp Obtain_quotient Inc_quotient: inc r21 rjmp Loop_quotient Obtain_quotient: add r18, r20 ret ; Subroutine to count up with debouncing Debouncing: Sampling_D: sbic PINB, 0 rjmp Sampling_D Pressed_D: sbis PINB, 0 rjmp Pressed_D call Delay10ms call Delay10ms sbis PINB, 0 rjmp Pressed_D ret

# EXPERIMENT 3:

- Answer the following questions:

  - Describe the connections of the modules in the experiment.

Connect Port A to the LCD similarly to Exp 1 and 2 Connect PORTB to Keypad Matrix Connect PORTD to LED

  - Is there any button debounce issue with the matrix keypad? If so, how is it handled?

All four columns are scanned at a pace that is far quicker than the average rate of key pushes and releases made by a human. This makes sure that any key presses may be accurately detected during the key scanning procedure. The approach used in this application to deal with the debounce issue entails periodically assessing whether the key has been pushed or released at regular intervals.

  - Present the source code of the program with comments.

.include "m324padef.inc" ; Include Atmega324pa definitions .org 0x0000 ; interrupt vector table rjmp start .org INT_VECTORS_SIZE .equ LCDPORT = PORTA ; Set signal port reg to PORTA .equ LCDPORTDIR = DDRA ; Set signal port dir reg to PORTA .equ LCDPORTPIN = PINA ; Set clear signal port pin reg to PORTA .equ LCD_RS = PINA0 .equ LCD_RW = PINA1 .equ LCD_EN = PINA2 .equ LCD_D7 = PINA7 .equ LCD_D6 = PINA6 .equ LCD_D5 = PINA5 .equ LCD_D4 = PINA4 .def LCDData = r16 message: .db "0xff nopressed",0 ; Replace with your application code ; ATmega324PA keypad scan function ; Scans a 4x4 keypad connected to PORTB ;C3-C0 connect to PB3-PB0 ;R3-R0 connect to PB7-PB4 ; Returns the key value (0-15) or 0xFF if no key is pressed start: ldi r16, 0xff out ddrc, r16 call LCD_Init loop: call keypad_scan out portc, r23 cp r23, r25 breq loop mov r25, r23 ldi r16, 0x01 ; Clear Display call LCD_Send_Command ldi r16, 0x80 ; Clear Display call LCD_Send_Command cpi r23, 0xff brne key_pressed ldi ZH, high(message) ; point to the information that is to be displayed ldi ZL, low(message) call LCD_Send_String rjmp loop key_pressed: cpi r23, 10 brlo less_than_A ldi r24, 7 add r23, r24 less_than_A: ldi r24, 48 add r23, r24 mov r16, r23 call LCD_Send_Data rjmp loop ; Replace with your application code ; ATmega324PA keypad scan function ; Scans a 4x4 keypad connected to PORTB ;C3-C0 connect to PB3-PB0 ;R3-R0 connect to PB4-PB7 ; Returns the key value (0-15) or 0xFF if no key is pressed keypad_scan: ldi r20, 0b00001111 ; set upper 4 bits of PORTB as input with pull-up, lower 4 bits as output out DDRB, r20 ldi r20, 0b11111111 ; enable pull up resistor out PORTB, r20 ldi r22, 0b11110111 ; initial col mask ldi r23, 0 ; initial pressed row value ldi r24,3 ;scanning col index keypad_scan_loop: out PORTB, r22 ; scan current col nop ;need to have 1us delay to stablize sbic PINB, 4 ; check row 0 rjmp keypad_scan_check_col2 rjmp keypad_scan_found ; row 0 is pressed keypad_scan_check_col2: sbic PINB, 5 ; check row 1 rjmp keypad_scan_check_col3 ldi r23, 1 ; row1 is pressed rjmp keypad_scan_found keypad_scan_check_col3: sbic PINB, 6 ; check row 2 rjmp keypad_scan_check_col4 ldi r23, 2 ; row 2 is pressed rjmp keypad_scan_found keypad_scan_check_col4: sbic PINB, 7 ; check row 3 rjmp keypad_scan_next_row ldi r23, 3 ; row 3 is pressed rjmp keypad_scan_found keypad_scan_next_row: ; check if all rows have been scanned cpi r24,0 breq keypad_scan_not_found ; shift row mask to scan next row ror r22 dec r24 ;increase row index rjmp keypad_scan_loop keypad_scan_found: ; combine row and column to get key value (0-15) ;key code = row*4 + col lsl r23 ; shift row value 4 bits to the left lsl r23 add r23, r24 ; add row value to column value ret keypad_scan_not_found: ldi r23, 0xFF ; no key pressed ret LCD_Init: ; Set up data direction register for Port A ldi r16, 0b11110111 ; set PA7-PA4 as outputs, PA2-PA0 as output out LCDPORTDIR, r16 ; Wait for LCD to power up call DELAY_10MS call DELAY_10MS ; Send initialization sequence ldi r16, 0x02 ; Function Set: 4-bit interface call LCD_Send_Command ldi r16, 0x28 ; Function Set: enable 5x7 mode for chars call LCD_Send_Command ldi r16, 0x0E ; Display Control: Display OFF, Cursor ON call LCD_Send_Command ldi r16, 0x01 ; Clear Display call LCD_Send_Command ldi r16, 0x80 ; Clear Display call LCD_Send_Command ret LCD_Send_Command: push r17 call LCD_wait_busy ; check if LCD is busy mov r17,r16 ;save the command ; Set RS low to select command register ; Set RW low to write to LCD andi r17,0xF0 ; Send command to LCD out LCDPORT, r17 nop nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN swap r16 andi r16,0xF0 ; Send command to LCD out LCDPORT, r16 ; Pulse enable pin sbi LCDPORT, LCD_EN ;nop ;nop cbi LCDPORT, LCD_EN pop r17 ret LCD_wait_busy: push r16 ldi r16, 0b00000111 ; set PA7-PA4 as input, PA2-PA0 as output out LCDPORTDIR, r16 ldi r16,0b11110010 ; set RS=0, RW=1 for read the busy flag out LCDPORT, r16 nop LCD_wait_busy_loop: sbi LCDPORT, LCD_EN nop nop in r16, LCDPORTPIN cbi LCDPORT, LCD_EN nop sbi LCDPORT, LCD_EN nop nop cbi LCDPORT, LCD_EN nop andi r16,0x80 cpi r16,0x80 breq LCD_wait_busy_loop ldi r16, 0b11110111 ; set PA7-PA4 as output, PA2-PA0 as output out LCDPORTDIR, r16 ldi r16,0b00000000 ; set RS=0, RW=1 for read the busy flag out LCDPORT, r16 pop r16 ret DELAY_10MS: ldi r21,80 ;1MC L1: ldi r20,250 ;1MC L2: dec r20 ;1MC nop ;1MC brne L2 ;2/1MC dec r21 ;1MC brne L1 ;2/1MC ret ;4MC LCD_Send_Data: push r17 call LCD_wait_busy ;check if LCD is busy mov r17,r16 ;save the command ; Set RS high to select data register ; Set RW low to write to LCD andi r17,0xF0 ori r17,0x01 ; Send data to LCD out LCDPORT, r17 nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop cbi LCDPORT, LCD_EN ; Delay for command execution ;send the lower nibble nop swap r16 andi r16,0xF0 ; Set RS high to select data register ; Set RW low to write to LCD andi r16,0xF0 ori r16,0x01 ; Send command to LCD out LCDPORT, r16 nop ; Pulse enable pin sbi LCDPORT, LCD_EN nop cbi LCDPORT, LCD_EN pop r17 ret LCD_Send_String: push ZH ; preserve pointer registers push ZL push LCDData ; fix up the pointers for use with the 'lpm' instruction lsl ZL ; shift the pointer one bit left for the lpm instruction rol ZH ; write the string of characters LCD_Send_String_01: lpm LCDData, Z+ ; get a character cpi LCDData, 0 ; check for end of string breq LCD_Send_String_02 ; done ; arrive here if this is a valid character call LCD_Send_Data ; display the character rjmp LCD_Send_String_01 ; not done, send another character ; arrive here when all characters in the message have been sent to the LCD module LCD_Send_String_02: pop LCDData pop ZL ; restore pointer registers pop ZH ret